WHAT GOES INTO MAKING A MUG?

1.	Drive to Sheffield, MA and pick up the clay

2. 	Unload clay into basement or garage.

3.	Weigh out amount needed.

4.	Wedge (knead) clay.

5.	Center clay on potters’ wheel and throw the mug shape.

6.	Remove from wheel and let dry to leather hard, several hours to 24 hours, depending on humidity.

7.	Put mug back on wheel and trim excess clay away, carve if desired.

8.	Pull a handle.

9.	Let handle stiffen.

10.	Attach handle to trimmed mug.

11.	Cover handle in wax to slow dry, or cover entire mug to dry slowly.

12.	Uncover mug and let dry to bone dry…maybe a few days to a week depending on humidity. If mug cracks at this point, re-cycle clay and start over at step 3.

13.	Bisque fire mug to 1830 degrees, about 12 hours. Allow kiln to cool before unloading (another 12-24 hours).

14.	If mug has cracked during this first firing, smash with hammer and go back to step 3…if no cracks, continue on.

15. 	Mix glaze or glazes for decorating mug.  Mixing a glaze may take 30-45 minutes (plus clean-up).

16.	Put wax on the bottom of the mug so glaze does not stick to kiln shelf.

17.	Choose design and glaze mug.

18.	Let glaze dry thoroughly before firing again.

19.	Apply kiln wash to kiln shelves before loading glazed pieces.

20.	If glaze scratches or gets bumped on it’s journey to the kiln, scrape off glaze, wash pot, let dry and start back at step 17.

21. 	Fire glazed mug to approximately 2230 degrees (cone 6). This takes another 8-12 hours.

22.	Wait approximately 12-24 hours for kiln to cool to under 200 F before opening.

23.	Remove mug and inspect closely.  If cracked, smash with hammer and start over at step 3.

24.	If mug has miraculously survived to this point, clean sharp bits off bottom with a grinding stone or emery paper, and put out to for sale.

25.	Last, but certainly not least instruction, try not to attack the foolish person who innocently asks, “WHY DOES THIS MUG COST SO MUCH?”

AND, let us not forget all the hidden steps NOT numbered in list above:

- Answer phone

- Pay utility company

- Develop new glazes

- Come up with new ideas for pots

- Pay the utility company

- Re-cycle clay

- Pay the utility company

- Think up marketing strategies

- Clean studio

- Pay the utility company